APG – UC118i

Architectural and Engineering (A&E) Specifications

| Code | Revision | Date | Comment | Author |
| --- | --- | --- | --- | --- |
| DOP2021 | A | 2025-03-05 |  | MC |

UNILINE Compact Series

UC118i

The loudspeaker shall be a 18 inches dual chamber band pass subwoofer designed to be stacked or hanged. The loudspeaker shall include a 18 inches Neodymium driver especially designed to broadcast very low frequencies.

The maximum power handling shall be 3600W. The frequency range of the subwoofer shall be 25Hz to 80Hz on axis (-10dB) with max SPL of 140dB at 1 meter. The subwoofer shall be omnidirectional. The loudspeaker impedance shall be 8 Ohms.

The loudspeaker enclosure shall be made of 15mm Finnish Birch plywood and painted with impact resistant aquarethane paint. The front grid shall be made of acoustically transparent perforated steel. The loudspeaker enclosure shall be designed for indoor use. The loudspeaker shall have six (6) integral cut-out handle. The loudspeaker shall integrate a four points hardware for rigging.

The unit shall be available in black color (RAL9005).

The unit shall have two Neutrik Speakon NL4 terminal block with a Loop-through at the rear and  two Neutrik Speakon NL4 terminal block with a Loop-through at the front for the cardioid mount. The connector shall be designed for cable from 1.5mm2 to 4mm2 section. The manufacturer shall offer five (5) years warranty.

The high-performance indoor loudspeaker shall meet or exceed the following specifications:

The loudspeaker shall be APG UC118i.