NESS Documentation

Contents table

Installation	2

Windows:	2

Mac :	2

NESS Overview:	3

Sessions / Snapshots	5

Master panel	5

Audio settings	6

Design	7

Loading a background image	7

Speaker groups configuration	7

Audio settings of the speaker group	8

Sources	9

Source mixer:	10

Position controls	10

Output mixer	12

Reverb	13

Global Display	14

Snapshots	14

Settings	15

Midi settings	15

OSC Settings	16

Spatialization Settings	16

TouchOSC  Remote Control	17

Global View :	18

Sources Positions :	18

Source Mixer :	19

Snapshots :	20

Compatibility	20

# Version

Documentation for NESS v1.2.2

# Installation

NESS is free and can be downloaded from the dedicated website at this address:

https://ness-apg.com/

To create your license for NESS, please fill the form at this address:
https://www.arbane-groupe.com/en/blog.html?view=item&id=2

You must enter a valid email address to receive the license key for the software.

## Windows:

- Just launch the installer NESS_v1.2.2_win_setup.exe and follow the instructions.

- At the first launch of the application, a popup asks you for the license file, just select the file NESS_license.txt you received by mail.

- At the first launch, you must authorize the software to access private networks to control the spatialization from TouchOSC or Ableton Live

Mac:

- Unzip the archive.

- Launch the software.

- At the first launch of the application, a popup asks you for the license file, just select the file NESS_license.txt you received by mail. An internet connection is required to enable the software.

# NESS Overview:

Master Panel:

- Saving and loading sessions

- Saving and loading Snapshots

- Audio settings

- Main volume and mute

- Window Zoom

- Quit NESS

Menus:

- Speakers configuration

- Sources control

- Reverb settings

- Global display

- Snapshots management

- App Settings

- Help

Graphical area: depending on the active menu

- Design

  - Settings of the background image

  - Speakers position

  - Binaural listening position

- Sources :

  - Sources static positions

  - Spatialization mode: constant intensity, distance dependent gain

  - Dynamic moves

- Reverb

  - Display of all the objects (sources and speakers)

- Global display / Settings / Help

  - Display of all the objects (sources and speakers)

Mixer: depending on the selected menu

- Design

  - Gains / delays / phase of speaker groups and binaural

  - Routing matrix of speaker groups and binaural

- Sources

  - Source selection

  - Name of the source

  - Input channel selection

  - Gain / mute / solo

- Reverb

  - Input gain / mute of source signals

Settings: depending on the selected menu

- Design

  - Configuration of the stage

  - Loading a background image

  - Resizing the image

  - Transparency of the image

  - Scaling

  - Speaker groups selection

  - Output Matrix

  - Link of speaker groups

- Sources

  - Send faders for the speaker groups and binaural

  - Processing mode: spatialized or directly routed

  - Delays activation

  - Delays and gains monitoring

  - Send faders for the reverb

  - Monitoring of distance dependent reverb gain

- Reverb

  - Reverb bus color

  - Impulse response choice

  - Send faders for the speaker groups & binaural

  - Reverbs level monitoring

  - Configuration of the distance dependant reverb law

- Global Display

  - Name of the sources and speakers

  - Show / hide each source and speaker group

  - Open / close the 3D window

- Snapshots

  - List of the snapshot in the session

  - Select / fire a snapshot

  - Snapshot name

  - Saving the current snapshot

  - Saving the parameters in a new snapshot

  - Move snapshots in the list

- Settings

  - Midi Mapping

    - Loading a mapping file

    - Saving the mapping in a mapping file

    - Table of midi controls and the mapped UI functions

    - Selection of the midi device

    - Monitoring of incoming midi messages

  - OSC Settings

    - Display the current IP of the software (requires Java runtime environment)

    - Monitoring of the incoming messages

    - OSC rate

    - Sending OSC messages to other devices

  - Speakers spatialization settings

    - General Focus (equivalent to DBAP rolloff)

    - Atmospheric absorption activation

    - Smoothing of the delays

    - Spatial blur

    - Minimum delay mode

  - Binaural settings

    - Distance attenuation

    - Minimum distance before attenuation

  - Help

    - General information

    - Links to websites

    - Links to resources

      - Online documentation

      - Tutorials

      - Support contact

      - NESS community

      - FAQ

## Sessions / Snapshots

This panel allows to load a session file and save or modify snapshots of the session

Nes session: creates a blank session

Load: load a session file (.json)

Save as: save the current state of the software in a new empty session file

/!\ the created session file does not have any snapshot, a first snapshot must be created right after the saving of the new session

New snapshot: create a new snapshot from the current state of the UI in the open session

Rename snapshot: change the name of the selected snapshot

Save snapshot: save the current state of the software in the current snapshot (/!\ this will overwrite the snapshot)

## Master panel

State of the audio engine . To avoid overshoots on high volume PA systems, when the audio engine is turned on, the main mute is automatically activated.

Audio settings: opens the audio settings window

CPU consumption: displays the CPU load, over 60% of load, some artifacts can be audibles

Master Mute: cuts all the output channels

Master fader + Master vu meter : allows to tune the volume of all output channels

Window zoom: allows to adapt the size of the UI to the screen resolution. Click on « window zoom » to automatically adapt the UI to your screen.

### Audio settings

This window allows to tune the audio engine parameters of NESS.

See the dedicated documentation.

## Design

### Loading a background image

Configuration of the stage: choose your use case:

- Frontal stage:

  - must be used for theaters or concert stages where all speakers are aligned on the front of the stage.

  - The binaural listening position is set centered at the bottom of the graphical area

- Surround stage:

  - must be used for configuration where the sound sources can be spatialized at 360°.

  - The binaural listening position is set in the center of the graphical area.

- 180° stage:

  - must be used for immersive configurations where the listener should hear sources all around him but not behind.

  - The binaural listening position is set centered at the bottom of the graphical area

Load background image: allows to load a background image (venue plan)

Image size: resize the plan to adapt it to the graphical area

Opacity: allows to the transparency of the image

To center the image, keep ctrl + shift and the left button of the mouse pushed and move the mouse in the graphical area

Click on « Define Scale » to define the scale of the loaded image. Then draw a line in the graphical area between two points for which you know the distance. Enter the distance in meters in the dialog window.

To ensure a good readability of the UI, use black and white images in negative. If the spatialization zone has an orientation (the audience is on one side and the audio objects on the opposite side), it is recommended to have the audience on the lower side of the graphical area to keep the compatibility with other spatialization tools (OSC control…)

The speaker groups buttons selects the groups for the edition of the speakers positions and routing

Show Speakers: this button allows seeing/hiding all speakers on the graphical area

Open Output Matrix: opens the matrix to route the speakers signals to each of the physical outputs of the soundcard

Link speaker groups:
If 8 speakers per group are not enough for your setup, you can create a bigger group just by linking several groups. Just check the groups you want to link.
Once linked, the parameters of the linked groups are shared for all speaker groups in the link. If you change the color, edit the gain, mute one the subgroup… all other subgroups will see their parameters changing the same way.

### Speaker groups configuration

A speaker group has a maximum of 8 speakers. 4 groups are available.

Click on the button of the group to configure to display the UI for the creation and placement of the speakers

Clear: reset the group

Color: change the color of the group, the color will be applied to all faders sending signal to the group outputs, and to all objects representing the speakers in the graphical area.

Create one speaker: create on speaker at the coordinates entered in the numboxes (speaker x, speaker y)

Create line: creates a line of equidistant speakers. The number of speakers, the coordinates of the first and last speaker can be set in the commands below the button

Import: import a text file containing the coordinates of the speakers and creates the speakers at these coordinates. A group can only have 8 speakers, only the first 8 lines of the file will be imported.

Export: export in a text file the coordinates of the speakers of the group

Presets bar: when approaching the mouse to the upper side of the graphical area, the preset bar is displayed

Load: loads a preset file for the current group of speakers

Export: export all the presets of the speaker group un a .json file

Save: save the current state of the speaker group in the current preset

New: save the current state of the speaker group in a new preset

Rename: rename the selected preset

Copy: copy the state of the selected speaker group in the clipboard. The parameters can only be pasted in another speaker group

Paste: loads the parameters of the clipboard in the current speaker group

### Audio settings of the speaker group

Routing:

The routing is done thanks to 4 matrixes for each speaker group. The columns correspond to the signals of each virtual speaker of the group (1 to 8), the rows correspond to the 32 available outputs (1 to 8, 9 to 16, 17 to 24 and 25 to 32)

Example: on the upper image, signals coming from the virtual speakers 1, 2 and 3 are routed to the outputs 17, 18 and 19 of the sound card

You can also use the Open Output Matrix button to route all signals at once from your speaker groups to the soundcard outputs

Clear: reset the corresponding matrix

Diag: check the diagonal of the corresponding matrix

Clear all: reset the 4 matrixes

Select: displays the configuration UI for a given speaker group

Name: the name of the group can be defined and will be displayed for the send faders in the sources and reverb panels

Mute: cut the audio on all speakers of the group. It also cuts the processing of the group, consider mute unused groups to lower the CPU load

Phase: invert the phase for all speakers of the group

Delay: add a common delay to all speakers of the group. The delay can be set in ms in the numbox

Fader: gain control for all the speakers of the group

Vu meter: displays the audio level for each speaker of the group

## Binaural configuration

NESS v1.2.2 has an embedded binaural engine.

The controls are the same than the speaker groups
As for the speaker groups you can see the listening position on the graphical area by selecting the binaural group in the mixer

The listening position is set depending on your stage configuration:

- Frontal stage:

  - The binaural listening position is set centered at the bottom of the graphical area

- Surround stage:

  - The binaural listening position is set in the center of the graphical area.

- 180° stage:

  - The binaural listening position is set centered at the bottom of the graphical area

⚠ the binaural processing has a minimum latency of 42 ms. This does not depend on the audio settings.

## Sources

The sources panel allows to control the positions, moves, levels, reverbs and spatialization options for each of the 16 available virtual sources.

### Source mixer:

Select: display the controls for the selected source. The source track is highlighted

Name: the name of the virtual source can be defined. The name will be displayed on the graphical area, in the Reverb panel and in the global display panel.

Input: select the audio input of the sound card for the virtual source

Mute: cut the signal of the sound source. This also cuts the processing of the source, consider muting all unused source to lower the CPU load.

Solo: cut the signal of all sources for which the solo button is not enabled. A clear solo button is available to deactivate all solo buttons of all sources

Fader: gain control of the incoming signal

### Position controls

Color: change the color of the source, the color will be applied to all representation of the source and its trajectory on the graphical area, on the associated faders in the source mixer and reverb mixer

Free move: moves the source freely in the graphical area. The source can be placed with the mouse. If the source is controlled by an external software (OSC automation…), this option must be checked

Trajectory: edit a trajectory for the source

Width: control for the perceived width of the source

Constant gain: allows to spatialize the sound source with a constant intensity, whatever the distance to the speaker group. If deactivated, the more distance between the source and the speakers, the lower will be the signal.

Presets bar: when approaching the mouse to the upper side of the graphical area, the preset bar is displayed

Load: loads a preset file for the selected source

Export: export all the presets of the source un a .json file

Save: save the current state of the source in the current preset

New: save the current state of the source in a new preset

Rename: rename the selected preset

Copy: copy the parameters of the selected source in the clipboard. The parameters can only be pasted for another source.

Paste: loads the parameters of the clipboard in the current source editor

#### Trajectory management

To create a trajectory, just click in the graphical area to create a point.
Shift + click = delete the point 
the points can be moved with the mouse

Clear: reset the trajectory.

Play: trigger the movement

Pause: stops the movement. Click again to play again the movement from the last position.

Loop: plays the movement repeatedly.

Stop: stops the movement and places the source at the beginning of the trajectory.

Movement direction: choose the movement direction or a round trip movement.

Duration: total duration of the movement.

Position on trajectory: set the position on the trajectory
example: a value of 2.5 will place the source at the middle of the segment between points 2 and 3 of the trajectory.

Locking the source position: if you enables the caps lock key in free move mode, it will lock the edition of the source position from the graphical editor. The source can still be moved with OSC messages or by using trajectories.

Quick tip: by placing the trajectory points at adequate positions of the virtual stage, it allows to place the source with only one parameter.

0 = in the front on the left
1 = centered in the front 
2 = in front on the right
3 = in the middle on the right
4 = in the rear on the right
5 = in the rear centered
6 = in the rear on the left
7 = in the middle on the left
8 = centered in the middle

#### Programming movements:

You can use the trajectory tool to control both position and speed of the source. The same amount of time is taken by the source to from one point to the next one. Close points will decrease the source speed, far points will accelerate the movement. The total duration is set in the « duration » numbox.

### Output mixer

The output mixer sends the signal to the 4 groups of speakers, the binaural engine and to the reverb engine. The spatialization of the sources is also controlled here.

Mute: cut the sound of the source on the bus. This cuts also the processing; you can consider muting unused busses to lower the CPU load

Fader: controls the send gain to the busses

Processing mode: allows to spatialize the source on a given speaker group (« spatialized ») or to directly route the signal to each speaker of the group (« direct routing »)

Nota: the gain computation of the source is by default independent for each group of speakers. It depends on the number of speakers in the group and the distance from the source to each speaker of the group. The groups are limited to 8 speakers, but you can link several groups to spatialize the source on bigger speaker arrays. See the design section for more information;

Gains and delay monitoring: visualization of the spatialization settings.

Delays activation: the delays can be activated or not. For static sources, it is recommended to activate the delay to strengthen the spatialization effect. For sources which are subject to move, the delays will be interpolated, which can create a doppler effect. This effect can be attenuated by enabling the minimal delay mode and increasing the time constant of the smoothing in the settings panel, or the delay can be deactivated.

Direct routing: the signal of the source is directly routed to the selected speakers of the group thanks to the matrix.

## Reverb

The reverb algorithm is based on convolutions. A set of impulse responses is given with the software, but other IR files can be added in the preset/IR folder of the application.

IR selection: selects an impulse response of the folder /presets/IR. You can also drag and drop a folder on the menu to access the IR files of the folder.

Color: change the color of the reverb, the color will be applied the gains faders of the reverb busses of each source

A representation of the loaded IR and the reverberation time are displayed.

To recreate the distance effect, the UI allows to define a distance dependent reverb law that is common for all sources and ensure a coherent acoustic space rendering. The lower right section allows to define the amount of reverb depending on the source distance to a reference point. The reference point is set as follow:

- In frontal stage mode, the distance of each source is computed from the lower side of the graphical area.

- In surround mode, the distance of each source is computed from the center of the graphical area, which is also the binaural listening position.

- In 180° mode, the distance of each source is computed from the middle of the lower side of the graphical area, which is also the binaural listening position.

The distance law is defined with the following parameters:

Reverb close: sets the gain of the reverb in dB for a source that is close to the reference position.

Reverb Far: sets the gain of the reverb in dB for a source that is far from the reference position.

The upper left section allows to set the amount of reverb in each of the speaker groups and in the binaural engine.

The lower left section indicates the amount of reverb for each source.

The mixer section allows to set the generic amount of reverb for each source. The gain is added to the distance dependent gain. Those faders have the same values than the rev send gains that are sets on the sources views.

## Global Display

The global display page displays all the sources and speakers and the inputs and outputs audio meters. Sources and groups of speakers can be shown or hidden by clicking on their labels. The muted sources and speaker groups are automatically hidden. Sources can be moved directly from the graphical area.

The open and close 3D view button opens a 3D representation of the scene in another window.

## Snapshots

The snapshots panel allows to manage the snapshots of the session

Snapshot table: visualization of the current snapshot (blue). Selection of a snapshot to modify (yellow)

Fire snapshot: loads the selected snapshot (in yellow)

Rename snapshot: rename the selected snapshot (in yellow)

Delete snapshot: deletes the selected snapshot (in yellow)

Save snapshot: save the current snapshot (in blue)

New snapshot: create a new snapshot just after the current one (in blue)

Move Up: moves the snapshot up in the list.

Move down: moves the snapshot down in the list.

## Settings

### Midi settings

Most of the UI controls can be mapped to a midi controller. The UI supports midi learn from any midi device

Press AltGr + M on a given screen of the application to map a control to a midi controller

Click on the control to map and click on “Enable for mapping”

Use the controller button that must be associated to the UI control

Press Escape to quit the mapping mode

Press AltGr + k on a given screen of the application to map a control to a keyboard key

Click on the control to map and click on “Enable for mapping”

Press a key on the keyboard to assign it to the UI control

Press Escape to quit the mapping mode

Load mapping: loads a saved mapping file

Save mapping: saves all mappings in a mapping file

Show mapping window: displays the mapping table for midi device and keyboard

Midi device: select a midi device in the list

Incoming midi message: display the incoming midi message values

The midi and keyboard mappings are automatically saved in the software.

### OSC Settings

NESS can be controlled with the OSC protocol with the TouchOSC application and the available here. NESS also supports the ADM OSC protocol for the control of the sources

NESS receives osc messages on the port 4000

When the TouchOSC app is connected to NESS The message « TouchOSC connected » is displayed in green

Receiving OSC: turns on or off the processing of incoming OSC messages

Network interface: displays the IP address of the selected network interface.
Nota: this requires Java runtime engine to be installed

OSC rate: defines the rate at which the incoming message are processed. This can increase or decrease the CPU load.

Incoming OSC message: displays the OSC messages on the port 4000 received by the software

Sender: NESS can also send OSC messages to up to 4 other devices

Destination device name: name of the device to control

IP address: IP address of the mobile device on which TouchOSC is installed or IP address of the application to control

Port: UDP port on which NESS sends OSC messages

### Speakers Spatialization Settings

General Focus or DBAP Rolloff (% - dB/ double distance): sets the level decrease law for the acoustic model of the DBAP. 36% / 6 dB is a usual value to simulate the acoustic propagation and is required for WFS (wavefield synthesis) rendering.

A low value will spread the sources and can be useful on large speaker setups.
A higher value will lead to less speakers playing the sound of one source at the same time, which can enhance the precision of the spatialization for small 360° setups.
A too low value can lead to weak precision in the spatialization perception
A too high value can stick the sources to the positions of the speakers, and produce position jumps when the sources moves from one speaker to another.

Atmospheric Absorption: activates or deactivates the high frequency attenuation depending on the distance.

Delay smoothing: smoothing coefficient for the delays. For sources with quick movements, this can create unwanted doppler effect (frequency transposition). The delays are interpolated and smoothed to avoid high pitch transitions. By increasing the value, the transitions are smoothed, but the delays will be less corelated to the source position, which can results in a slightly less precise localization.

Spatial Blur: this parameter can smooth the intensity of the speakers when the sources are very close to the speaker position. Try a value close to the spacing of two consecutive speakers.

### Binaural Settings

Distance attenuation: this parameter allows to set the decreasing of the sound intensity when a source is moving away from the listening position.

Min attenuation distance: below this distance, the sound sources are not attenuated.

⚠ the binaural processing has a minimum latency of 42 ms. This does not depend on the audio settings. This feature is then not suitable for real time in ear monitoring.

## TouchOSC Remote Control

The TouchOSC application can be downloaded at this address: https://hexler.net/touchosc

The documentation is available at this address: https://hexler.net/touchosc/manual/getting-started

Open the file APG_NESS.tosc with the TouchOSC application

To connect the application to NESS:

- Open the menu “connections” (button )

- Open the OSC tab

- Configure a UDP connection

- Enter the IP address of the computer on which NESS is launched

- Set the sending port to 4000

- Set the receiving port to 40001

- The IP address of the device on which TouchOSC is running must be entered in the Settings panel of NESS

- Launch the application button

- In the snapshots tab, click on « update snapshot table », the list of snapshots displayed in NESS must be loaded

- Select a snapshot from the TouchOSC app

- Check the sources positions in the Global view tab. It must be the same as in NESS

## Global View:

Display the source positions + their labels

## Sources Positions:

The Sources Position tab allows to control the position of each source with XY pad.

The sources are split on two tabs: sources 1 to 8 and 9 to 16

A slider allows to set the width of the source

The button triggers the movement programmed in NESS

### Source Mixer:

The Source Mixer tab controls the gains of the sources and the master fader

For each source, the following controls are available:

- Gain fader

- Mute button

- Solo Button

The main controls are the following

- Master fader

- Master mute

- Clear solo

### Snapshots:

The snapshots tab allows to load snapshots of the current session from the mobile device.

The current snapshot is highlighted in yellow

The « Update Snapshot table » button forces the upload of the snapshots of the session

Troubleshooting:

- If the app controls NESS but the changes in NESS are not updated in TouchOSC, check the IIP address and the UDP port entered in the Settings panel in NESS, and the UDP port in the “Connections” menu of TouchOSC

- If the application does not communicate with NESS, check that the label « ToucOSC connected » is displayed in green in the Settings panel. If it is not the case, check the IP address in the Connections tab in TouchOSC

## Compatibility

NESS is compatible with the following applications:

- Windows 10 and 11

- Mac OS 12.5 and newer versions, for Intel and Apple Silicon platforms.

The software is design to be adaptable to all kind of computers, with different CPU resources.

To optimize the CPU load:

- Deactivate the unused sources (Mute buttons on the mixer sources)

- Deactivate the unused groups (Mute buttons in the Design panel)

- Deactivate the atmospheric absorption in the Settings Tab

- Deactivate the unused reverb busses: Mute buttons over the reverb fader in the sources panels, or on the reverb mixer.

- Increase the size of the buffers (I/O vector size and signal vector size)

- Decrease the OSC rate.